Comparative Study Of Modren Instruction Set Architectures

line 1: 1st Waleed Tawfiq
line 2: Department of Computer Engineneering
line 3: University Of Jordan
line 4: Amman,Jordan
line 5: WLY0231046@ju.edu.jo

line 1: 4th Motaz Majdi
line 2: Department of Computer Engineneering
line 3: : University Of Jordan
line 4: Amman,Jordan

line 5: MAT0189290@ju.edu.jo
line 1: Motaz EL-Mashaleh
line 2: Department of Computer Engineneering
line 3: : University Of Jordan
line 4: Amman,Jordan
line 5: Mat1260067@ju.edu.jo

line 1: 3rd Mohammad Awwad
line 2 Department of Computer Engineneering
line 3: : University Of Jordan
line 4: Amman,Jordan
line 5: MHM2220231@ju.edu.jo

# Introduction

An instruction set architecture (ISA) is the abstract interface between a processor’s hardware and the software it runs. It defines the supported instructions, data types, registers, memory access models, and I/O handling, enabling software compatibility across different implementations of a processor family [1].

The isa provides a common foundation that allows software to run on different processor implementation while still giving hardware designers the freedom to improve performance and efficiency [1].

Different ISAs have been developed because architecture features that are useful for one application domain may not provide the same benefits for another [2].

ISA design is important because the choices made in its design can affect processor performance, hardware, complexity, energy use, and software development.

In this report we will compare RISC -V with other ISAs to examine their main differences and how their design choices affect different computing applications.

# Overview of the selcted ISAs

## RISC-V

RISC-V follows a modular and extensible design philosophy, where a simple base instruction set can be combined with additional standard or custom extensions depending on the needs of the system [2].

It can be used in a wide range of application domains, embedded and IoT devices, automative systems, data centers, industrial applications [3].

RISC-V has simple and regular instruction set. With base versions for 32 bit and 64-bit systems, it also supports optional extensions that can add extra features when needed [2].

It’s an open-Standard ISA that anyone can use [4].

## X86-64

Interestingly x86-64 extends the older x86 architecture to support 64 bits of computing while keeping compatibility with older x86 software [5].

It is mainly used in personal computers, laptops, desktops servers and data center [6].

It’s a 64-bit architecture with a large instruction set and support for 64 operations [7].

At the same time, its licensed architecture is mainly developed and maintained by AMD and intel [8].

C.MIPS32

In Contrast, MIPS32 follows a RISC design with simple and regular instruction set (32-bit design) [9].

It is commonly used in embedded devices, including consumer systems and automative [10].

MIPS uses load/store design and has 32 general registers [11].

Unlike RISC-V, MIPS32 isn’t open ISA and is used under A licensing model [12].

D.ARMv9-A

What makes ARMv9-A stand out is the focus on performance, efficiency and security [13].

ARMv9-A is used in smartphones, computers, cars and very high-performance systems [14].

It is a 64-bit architecture with support for both vector and matrix operations [15].

On the other hand, ARMv9-A is licensed architecture and isn’t open like RISC -V [16].

# INSTRUCTION SET COMPARSION

Table I compare the main instruction -set features of the four ISAs [7],[17],[18],[19]

| Feature | RISC-V | X86-64 | MIPS32 | ARMv9-A |
| --- | --- | --- | --- | --- |
| Instruction length – encoding | Mostly 32-bit instruction, with optional 16-bit compressed instruction | Use variable length instruction, up to 15 bytes | MIPS32 mostly uses 32-bit instructions but it can also be 16-bit instructions | Uses fixed 32 -bit instructions in A64 |
| Instruction format | Uses simple instruction formats like R, I, S, B, U and J | Uses different instruction formats with an opcode and optional extra fields | Uses 3 main formats: R -type, I -type and J -type | Uses different 32-bit formats for different types of instructions |
| Register organization | 32 int registers, with 0x always on zero | Has 16 general purpose registers in 64-bit mode | MIPS32 has 32 registers can be used for the general operations | It has 31- general purpose registers in 64-bit mode |
| Addressing models | Uses a base register with an offset to access memory | Uses base and index registers with an offset and access memory | Uses a register and an offset to access memory | Uses a register with an offset to access the memory |
| Arithmetic and logical instructions | Support basic arithmetic and logical operations such as ADD, SUB, OR XOR, AND | Supports a wider range of arithmetic and logical instruction, with some able to use memory directly | Support basic arithmetic and logical operations such as ADD, SUB, OR XOR, AND | Support basic arithmetic and logical operations such as ADD, SUB, OR XOR, AND |
| Memory access instructions | Uses load and store instructions to move data between memory and registers | Can move data between register and memory using certain instructions like Mov | Uses load and store instructions to move data between memory and registers | Uses load and store instructions to move data between memory and registers |
| Branch and control flow | Uses conditional branches and jump instruction for control flow | Uses jump, branches and CALL/RET to control program control | Uses branches and jumps to change where the program goes next | Uses branches and jumps to change where the program goes next |

Table I. INSTRCUTION SET COMPARSION

The comparison shows that the 4 ISAs take different approaches to do the same basic Tasks ,RISC-V and MIPS32 keep their instruction format more regular and simple(straightforward),while x86-64 uses variable length instructions and a more flexible encoding style [7] ,[17] ,[18].ARMv9-A also uses fixed 32 bit instructions but it adds more modern features for different types of processing .Another clean difference is memory access :RISC-V and MIPS32 rely on load and store instructions ,while x86-64 allows memory to be used more directly in many instructions [7] ,[17] ,[18].Overall ,these differences show how each ISA makes its own trade-off between simplicity , flexibility ,and the features it provides [7],[17],[18],[19]

# ARCHITECTURAL ANALYSIS

Table II compares how the architecture choices of the 4 ISAs affect hardware design ,performance , and efficiency [7],[13,[15],[17],[18],[19],[20]

| Factor | RISC-V | X86-64 | MIPS32 | ARMv9-A |
| --- | --- | --- | --- | --- |
| Decoder complexity | Low-regular instruction format makes decoding easier | High-its instructions format has more parts to decode | Low-its simple instruction formats make the decoding easier | Low-its fixed 32-bit instruction make the decoding easier |
| Pipeline design | Simple-its regular instruction format makes pipeline design easier | Complex-the pipeline front end must handle variable length instruction | Simple -its regular instructions make the pipeline easier to design | Simple -its fix 32 bits instructions make the pipeline easier to design |
| Code density | Good-it can use shorter instructions to save space | Good-variable length instructions can help reduce code size | Good-it can use shorter 16-bit instructions to save space | Moderate-fixed 32 instructions may use more space |
| Complier optimization | Flexible – the complier can generate code based on available extensions | Flexible complier can adjust the code for different x86 processors | Good compliers can optimize MIPS32 code for better speed and smaller size | Good-its regular design helps compiler optimization |
| Processor performance | Scalable – can be implanted in both simple and high-performance processor | High-it is widely used in systems that need strong performance | Good-MIPS32 can give strong performance in embed systems | Good SVE2 helps speed up data processing |
| Energy efficiency | Efficient – can be customized for energy – efficient designs | Depend on design -it can be made power efficient | Good-MIPS32 can give good performance without suing too much power | Good-designed to balance performance and power usage |

Table II.ARCHITECTURAL ANALYSIS OF THE SELCTED ISAs

The comparison show how each ISA makes a different tradeoff between simplicity and flexibility .RISC-V ,MIPS32 and ARMv9-A rely on more regular instruction designs ,which generally make decoding and pipelining easier ,while x86-64 accepts more complexity in exchange for richer and more flexible instruction format [7],[17]-[20].

V.APPLIICTAION SUITABLITY

Table III compares the suitability of the four ISAs for different application areas [3],[6],[7],[10],[13]-[15].

| Application | RISC-V | X86-64 | MIPS32 | ARMv9-A |
| --- | --- | --- | --- | --- |
| Embedded systems | High-easy to customize | High-x86-64 has processors designed specifically for embedded systems but with high complexity | High-it works well for embedded and real time systems | Good-Works well in embedded systems that need high performance and efficiency |
| Mobile devices | Good-flexible enough for different types of devices | Good-can provide responsible performance and power efficiency in mobiles | Low-it is mainly designed for embedded systems | High-Work perfectly in phones because it gives high performance without using too much power |
| Desktop computers | Good-64-bit RISC-V application processors can run full operating systems | High- its highly used in computers and support strong purpose performance | Low -it’s mostly used for embedded systems, not desktops | High-it’s a great choice for pcs and laptops it gives high performance and use power efficient way |
| Servers | High- has standards designed specifically for server systems | High-its widely used in servers | Very low- it’s mostly used for embedded not servers | High- its great choice for severs because it combines high performance with good energy usage |
| IoT devices | High works well for IoT because it can be used in small and efficient processors | Good-can be used for IoT devices that need responsible processing | Good-it works well for IoT and gateways | Very high-it works well because it combines high performance, efficiency and security |
| HPC | High – it can be customized for high performance | High-are widely used for HPC | Low-it isn’t made for HPC | High works well, it gives high performance and energy saver |

TABLE III. APPLICTION SUITBALITY FOR SELECTED ISAs

This comparison shows that there is no single ISA that is best choice for every application’s RISC-V stands out for systems that benefit from flexibility especially in embedded and IoT devices [3].x86-64 remains strong for desktops, servers, where processing power is a major priority [6]. MIPS32 fits in better in simpler embedded systems [10]. while ARMv9-A offers a strong balance between performance and power efficiency [13]-[15].

VI..CONCLUSION

RISC-V stands out for its simple and flexible design. Its modular structure allows designers to use only features they need, making it a strong choice for embedded and customized systems [2],[3]. However, having different configurations can make software compatibility more challenging. for this reason, RISC-V can be preferred over the ISAs when customization and design flexibility are the main properties [2].

X86-64 takes a different path, instead of keeping the instruction set simple, it accepts more complexity to provide strong performance and combability [6],[7]. This makes decoding and hardware design more difficult, but it remains a strong option for desktops, servers, and demanding workloads [6], therefore x86-64 is preferred when performance and compatibly matters more than having a simple hardware design.

MIPS32 shows that a processor does not always need a complicated design to be useful, its simple and regular structure makes it a natural fit for embedded and real time systems [10],[11]. The tradeoff is that it offers fewer demanding desktops, servers, and high-performance workloads, so when the goal is straightforward and efficient embedded system rather than maximum computing power, MIPS32 becomes the more practical choice [10].

ARMv9-A is all about balance, it delivers strong performance while keeping power efficiency in mind [13],[15] that balance allows it to work well across very different systems, from smartphone and PCs to servers and high-performance machines [14]. Its advanced features make it more complexity, but when both speed and efficiency matter, ARMv9-A is often the more balanced [13]-[15].

##### References

Arm,’What is Instruction Set Architecture (ISA)? ’,Arm Glossary.[Online].Available: What is Instruction Set Architecture (ISA)? – Arm®

RISC-V International ,’Inttroduction,’The RISC-V Instruction Set Manuall,Volume I:Unprivliged Architecture.[online] Available: Introduction :: RISC-V Ratified Specifications Library

RISC-V international,’RISC-V Annual report 2025’.[Online].Available:https://riscv.org/wp-content/uploads/2026/01/RISC-V-Annual-Report-2025.pdf

RISC-V international ,’FAQ’ RISC-V International .[Online].Available: https://riscv.org/about/faq/

Advanced Mirco Devieces ,Inc.,AMD64 Architecture Programmer’s Manual,Volume 1 :Applictaion Programmimg ,Rev.3.25 ,publication No.24592 ,May.2026 .[Online].Available: AMD64 Architecture Programmer's Manual Volume 1: Application Programming (PUB) (24592) • Viewer • AMD Technical Information Portal

Intel Copporation,’what is x86 Architecture? A rpimer to the foundtaion of Modren Computing,’Intel newsroom,Oct 3,2025.[Online].Available: What is x86 Architecture? A Primer to the Foundation of Modern Computing - Newsroom

Intel Coportaion ,Intel 64 and IA-32 Archieture Software Developr’s Mauual,Combained,Volumes:1.2A-2D,3A-3D and  4.[Online].Avaliable: https://www.intel.com/content/www/us/en/developer/articles/technical/intel-sdm.html

Advanced Micro Devices ,Inc . and Intel Coportaion ‘Patent cross license Agreement’Nov .11 ,2009.[online].Available: https://ir.amd.com/financial-information/sec-filings/content/0001193125-09-236705/dex102.htm

Microchip Technology Inc.,’PIC32MZ Architecture’ Microship Developer Help.[online].available: https://developerhelp.microchip.com/xwiki/bin/view/products/mcu-mpu/32bit-mcu/PIC32/mz-arch-cpu-overview/architecture/?

Microchip Technology Inc.,’MIPS32 core based Microcontrollers (MCUSs)’,Microchip Technology.[online].Available: https://www.microchip.com/en-us/products/microcontrollers/32-bit-mcus/pic32m

Microship Technology Inc.,PIC32 MX Family Refrenace Manuel ,Section:1ntroduction ,DS61127C,2008.[Online].Available: https://ww1.microchip.com/downloads/en/DeviceDoc/61127C.pdf

MIPS Technologies ,Inc.,MIPS architecture for programmers ,Volume I-A:Introduction to the MIPS32 Architecture,Rev.3.02.Document No.MD0082.Mar.21,2011.[online].Available: MIPS32® Architecture Volume I: Introduction to the MIPS32® Architecture

Arm,’Arm’s Solution to the Future Needs Of Ai,Security and speciallized Computing is v9’,Arm newsroom,Mar.30.2021.[Online].Avalibale: https://newsroom.arm.com/news/arms-solution-to-the-future-needs-of-ai-security-and-specialized-computing-is-v9

Arm ,’CPU Archiecture’Arm.[online].Available: https://www.arm.com/architecture/cpu

Arm,’A profile Archiectures’,Arm.[online.]Available : https://www.arm.com/architecture/cpu/a-profile

Arm,’Licensing Arm Technology and Subscriptions,’Arm.[online].Available: https://www.arm.com/products/licensing

RISC-V International,’RV32I Base Intger Instruction Set,Version 2.1’RISC-V Raatified Specifcitions Library.[online].Available: https://docs.riscv.org/reference/isa/v20240411/unpriv/rv32.html

Microship Technology Inc.,PIC32 Family Reference Maunel,Section 50:CPU for Devices With MIPS32 microAptiv and M-class Cores ,DS6001192.[Online].Available: https://ww1.microchip.com/downloads/aemDocuments/documents/WSG/ProductDocuments/ReferenceManuals/PIC32_Family_Reference_Manual_CPU.pdf

Arn,Armv8-A instrcution set architecture,Issue 1.1,2020.[online].Available: https://documentation-service.arm.com/static/68de36a2587fd957f5efc967

E.Blem ,J,Menon ,T.Vijayaraghaven, and K.Sankaralingam ,’ISA Wars:Understanding the relevance of Isa Being RISC or CISC to prefromace ,Powwr,and Energy on Modren Archiectures’ACM transcations on Computer Systems ,vol.33,no.1,2015,doi:10.1145/2699682.[Online].Available: https://documentation-service.arm.com/static/68de36a2587fd957f5efc967